# Objective

In the present document will be described the services and equipment available in the different meeting rooms of the IAC as well as it's use, operation and connections to ease fixing any issue.

# Content Index

Índice de contenidos

Inventario de equipos-servicios

Equipos

AULA

Sala Trífida

Sala Omega

Sala 1 Biblioteca

Sala 2 Biblioteca

Sala 3 Biblioteca

Sala GTC

Sala Pleyades

Sala Burbuja

Servicios

Aula

Sala Trífida

Sala Omega

Sala1 Biblioteca

Sala2 Biblioteca

Sala3 Biblioteca

Sala GTC

Sala Pléyades

Sala Burbuja

Esquemas de conexiones

Aula

# Services-Equipment Inventory

## Equipment

### AULA

|  | Beamer Format 16:10 Native resolution 1920 x 1200 Brightness 6000 lumens Contrast relation 2000:1 |
| --- | --- |
|  | Powered Screen Powered, big format, wall mounted. 530x300cm Reinforced aluminium box Facing cloth with 0,6mm of thickness, opaque on backface Gain 1,2, Clasification M-1 |
|  | Laptop Dell Latitude e6430 Intel Core I5 8 gb Ram 320 gb HDD space 14” Screen with resolution 1366 x 768 Vga / Hdmi connection 1920 x 1080 Wifi DVD |
|  | Presenter Presenter Logitech R400 with red laser pointer. |
|  | Apple TV HDMI 1.43 Wi‑Fi 802.11ac MIMO Ethernet 10/100Base-T Bluetooth 4.0 Infrared receptor |
|  | Chromecast Compatible with HDMI y CEC Maximum display output resolution 1080p HDMI Wi-Fi Standards 802.11 b/g/n/ac (2.4 GHz o 5 GHz) |
|  | Videoconference Cámara EagleEye™, 720p o 1080 HDX microphones Cables and remote controller Connections H.264 / H.263++/ H.261/ H.234 |
|  | Streaming Epiphan Capture at 2K resolutions (2048×2048 or 2560×1600 wide screen) 4-card or 6-card versions depending on number of sources you have Each card has 1 HD (VGA/DVI/HDMI) port and 1 SDI (3G, HD and SD) port andone SD (S-Video) port, capture from all at once |

### Trífida Room

|  | TV 42” TV with VGA and HDMI connections |
| --- | --- |

### Omega Room

|  | Powered Screen Powered, big format, wall mounted. 230x200cm Reinforced aluminium box Facing cloth with 0,6mm of thickness, opaque on backface Gain 1,2, Classification M-1 |
| --- | --- |
|  | TV 42” TV with VGA and HDMI connections |
|  | Videoconference Zoom and Jabber connections H.264 / H.263++/ H.261/ H.239 |
|  | Beamer Format 16:10 Native resolution 1920 x 1200 Brightness 2000 lumens Contrast relation 2000:1 |
|  | Teleconference Conference cisco phone with speaker. |

### Room 1 Library

|  | TV 65” TV with VGA and HDMI connections |
| --- | --- |

### Room 2 Library

|  | TV 65” TV with VGA and HDMI connections |
| --- | --- |

### Room 3 Library

|  | TV 65” TV with VGA and HDMI connections |
| --- | --- |

### GTC Room

|  | Powered Screen Powered, big format, wall mounted. 230x200cm Reinforced aluminium box Facing cloth with 0,6mm of thickness, opaque on backface Gain 1,2, Classification M-1 |
| --- | --- |
|  | TV 42” TV with VGA and HDMI connections |
|  | Videoconference Zoom and Jabber connections H.264 / H.263++/ H.261/ H.239 |
|  | Beamer Format 16:10 Native resolution 1920 x 1200 Brightness 2500 lumens Contrast relation 2000:1 |
|  | Teleconference Conference cisco phone with speaker. |

### Pleyades Room

|  | TV 80” TV with VGA and HDMI connections |
| --- | --- |
|  | TV 42” TV with VGA and HDMI connections |
|  | Videoconference Zoom and Jabber connections H.264 / H.263++/ H.261/ H.239 |
|  | Apple TV HDMI 1.43 Wi‑Fi 802.11ac MIMO Ethernet 10/100Base-T Bluetooth 4.0 Infrared receptor |
|  | Chromecast Compatible with HDMI y CEC Maximum display output resolution 1080p HDMI Wi-Fi Standards 802.11 b/g/n/ac (2.4 GHz o 5 GHz) |
|  | Teleconference Conference cisco phone with speaker. |

### Burbuja Room

|  | Powered Screen Powered, big format, wall mounted. 230x200cm Reinforced aluminium box Facing cloth with 0,6mm of thickness, opaque on backface Gain 1,2, Classification M-1 |
| --- | --- |
|  | TV 42” TV with VGA and HDMI connections |
|  | Videoconference Zoom and Jabber connections H.264 / H.263++/ H.261/ H.239 |
|  | Beamer Format 16:10 Native resolution 1920 x 1200 Brightness 2500 lumens Contrast relation 2000:1 |

## Servicios

### Aula

Beamer. - HDMI and VGA input and a screening display. Laptop with Windows for the presentations

Videoconference. - Polycom videoconference system. Allows H323 protocol.

Streaming.- El servicio de Streaming dispone de una cámara de video sony y de un servidor de Streaming Epiphan Standalone VGA.

Streaming.- The streaming service have available a Sony videocamera and a Streaming Epiphan Standalone VGA server. Allow live restransmition in internet/intranet and/or the record of the meeting, seminars, talks. Also include it publishment in http://iactalks.iac.es/

### Trífida Room

Projection. - 42” TV with VGA input

Teleconference. – Cisco teleconference system

### Omega Room

Screening. - HDMI and VGA input and a screening display.

Videoconference. - Logitech videoconference system. Allows h323, Zoom and Jabber.

Teleconference. – Teleconference Cisco phone.

### Library Room 1

Projection. - 65” TV HDMI and Dport inputs (VGA with adapter that it’s in the CAU).

### Library Room 2

Projection. - 65” TV HDMI and Dport inputs (VGA with adapter that it’s in the CAU).

### Library Room 3

Projection. - 65” TV HDMI and Dport inputs (VGA with adapter that it’s in the CAU).

### GTC Room

Projection. - Beamer with HDMI, VGA inputs and screening display.

Videoconference. - Polycom videoconference system. Allows H323 protocol.

Teleconference. – Teleconference Cisco phone.

### Pléyades Room

Projection. – 80” TV with HDMI, VGA inputs.

Videoconference. - Logitech videoconference system. Allows h323, Zoom and Jabber.

Teleconference. – Teleconference Cisco phone.

### Burbuja Room

Projection. - Beamer with HDMI, VGA inputs and screening display.

Videoconference. - Polycom videoconference system. Allows H323 protocol.

Teleconference. – Teleconference Cisco phone.

# Connections scheme

### Aula

Inputs

- VGA Connection

- HDMI Connection

- Apple wireless connection for Mac OS and IOS

- Chromecast wireless connection for Windows and Android

- Sony video camera

- All connections support till 1920x1080 screen resolution

Outputs

- Beamer y Streaming: support till 1920x1080 screen resolution

- Videoconference Polycom resolution 720p